ATTACHMENT 1

Regulated Contaminants with PRIMARY DRINKING WATER STANDARDS

| Contaminant |  | Unit Measurement |  | MCL (AL) [MRDL] |  | PHG (MCLG) [MRDLG] |  | Typical Source of Contaminant |  | Health Effects Language |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ## Microbiological Contaminants |  |  |  |  |  |  |  |  |  |  |
| Total Coliform Bacteria (Total Coliform Rule) | MCL: For systems that collect less than 40 samples per month: No more than 1 positive monthly sample For systems that collect 40 or more samples per month: More than 5.0% of monthly samples are positive |  |  |  | (0) |  | Naturally present in the environment |  | Coliforms are bacteria that are naturally present in the environment and are used as an indicator that other, potentially-harmful, bacteria may be present. Coliforms were found in more samples than allowed and this was a warning of potential problems. |  |
| Fecal coliform and E. coli (Total Coliform Rule) | MCL: A routine sample and a repeat sample are total coliform positive, and one of these is also fecal coliform or E. coli positive |  |  |  | (0) |  | Human and animal fecal waste |  | Fecal coliforms and E. coli are bacteria whose presence indicates that the water may be contaminated with human or animal wastes. Microbes in these wastes can cause short-term effects, such as diarrhea, cramps, nausea, headaches, or other symptoms. They may pose a special health risk for infants, young children, some of the elderly, and people with severely compromised immune systems. |  |
| Fecal Indicator (E. coli) (Ground Water Rule) | 0 |  |  |  | (0) |  | Human and animal fecal waste |  | Fecal coliforms and E. coli are bacteria whose presence indicates that the water may be contaminated with human or animal wastes. Microbes in these wastes can cause short-term effects, such as diarrhea, cramps, nausea, headaches, or other symptoms. They may pose a special health risk for infants, young children, some of the elderly, and people with severely compromised immune systems. |  |
| Fecal Indicators (enterococci or coliphage) (Ground Water Rule) |  |  | TT |  | N/A |  | Human and animal fecal waste |  | Fecal indicators are microbes whose presence indicates that the water may be contaminated with human or animal wastes. Microbes in these wastes can cause short-term effects, such as diarrhea, cramps, nausea, headaches, or other symptoms. They may pose a special health risk for infants, young children, some of the elderly, and people with severely compromised immune systems. |  |
| Turbidity |  |  | TT |  | N/A |  | Soil runoff |  | Turbidity has no health effects. However, high levels of turbidity can interfere with disinfection and provide a medium for microbial growth. Turbidity may indicate the presence of disease-causing organisms. These organisms include bacteria, viruses, and parasites that can cause symptoms such as nausea, cramps, diarrhea, and associated headaches. |  |
| Giardia lambia, viruses, heterotrophic plate count bacteria, Legionella, Cryptosporidium |  |  | TT |  | HPC = N/A; Others = (0) |  | Naturally present in the environment |  | Inadequately treated water may contain disease-causing organisms. These organisms include bacteria, viruses, and parasites that can cause symptoms such as nausea, cramps, diarrhea, and associated headaches. |  |
| ## Radioactive Contaminants |  |  |  |  |  |  |  |  |  |  |
| Gross Beta Particle Activity |  | pCi/L |  | 50 (a) |  | (0) |  | Decay of natural and man-made deposits |  | Certain minerals are radioactive and may emit forms of radiation known as photons and beta radiation. Some people who drink water containing beta and photon emitters in excess of the MCL over many years may have an increased risk of getting cancer. |
| (a) Effective 6/11/2006, the gross beta particle activity MCL is 4 millirems/year annual dose equivalent to the total body or any internal organ. 50 pCi/L is used as a screening level. |  |  |  |  |  |  |  |  |  |  |
| Strontium-90 |  | pCi/L |  | 8 |  | 0.35 |  | Decay of natural and man-made deposit |  | Some people who drink water containing strontium-90 in excess of the MCL over many years may have an increased risk of getting cancer. |
| Tritium |  | pCi/L |  | 20,000 |  | 400 |  | Decay of natural and man-made deposits |  | Some people who drink water containing tritium in excess of the MCL over many years may have an increased risk of getting cancer. |
| Gross Alpha Particle Activity |  | pCi/L |  | 15 |  | (0) |  | Erosion of natural deposits |  | Certain minerals are radioactive and may emit a form of radiation known as alpha radiation. Some people who drink water containing alpha emitters in excess of the MCL over many years may have an increased risk of getting cancer. |
| Combined Radium 226 & 228 |  | pCi/L |  | 5 |  | (0)(b) |  | Erosion of natural deposits |  | Some people who drink water containing radium 226 or 228 in excess of the MCL over many years may have an increased risk of getting cancer. |
| (b) If reporting results for Ra-226 and Ra-228 as individual constituents, the PHG is 0.05 pCi/L for Ra-226 and 0.019 pCi/L for Ra-228. |  |  |  |  |  |  |  |  |  |  |
| Uranium |  | pCi/L |  | 20 |  | 0.43 |  | Erosion of natural deposits |  | Some people who drink water containing uranium in excess of the MCL over many years may have kidney problems or an increased risk of getting cancer. |
| ## Inorganic Contaminants |  |  |  |  |  |  |  |  |  |  |
| Aluminum |  | ppm |  | 1 |  | 0.6 |  | Erosion of natural deposits; residue from some surface water treatment processes |  | Some people who drink water containing aluminum in excess of the MCL over many years may experience short-term gastrointestinal tract effects. |
| Antimony |  | ppb |  | 6 |  | 20 |  | Discharge from petroleum refineries; fire retardants; ceramics; electronics; solder |  | Some people who drink water containing antimony in excess of the MCL over many years may experience increases in blood cholesterol and decreases in blood sugar. |
| Arsenic |  | ppb |  | 10 |  | 0.004 |  | Erosion of natural deposits; runoff from orchards; glass and electronics production wastes |  | Some people who drink water containing arsenic in excess of the MCL over many years may experience skin damage or circulatory system problems, and may have an increased risk of getting cancer. |
| Asbestos |  | MFL |  | 7 |  | 7 |  | Internal corrosion of asbestos cement water mains; erosion of natural deposits |  | Some people who drink water containing asbestos in excess of the MCL over many years may have an increased risk of developing benign intestinal polyps. |
| Barium |  | ppm |  | 1 |  | 2 |  | Discharge of oil drilling wastes and from metal refineries; erosion of natural deposits |  | Some people who drink water containing barium in excess of the MCL over many years may experience an increase in blood pressure. |
| Beryllium |  | ppb |  | 4 |  | 1 |  | Discharge from metal refineries, coal-burning factories, and electrical, aerospace, and defense industries |  | Some people who drink water containing beryllium in excess of the MCL over many years may develop intestinal lesions. |
| Cadmium |  | ppb |  | 5 |  | 0.04 |  | Internal corrosion of galvanized pipes; erosion of natural deposits; discharge from electroplating and industrial chemical factories, and metal refineries; runoff from waste batteries and paints |  | Some people who drink water containing cadmium in excess of the MCL over many years may experience kidney damage. |
| Chromium |  | ppb |  | 50 |  | (100) |  | Discharge from steel and pulp mills and chrome plating; erosion of natural deposits |  | Some people who use water containing chromium in excess of the MCL over many years may experience allergic dermatitis. |
| Copper |  | ppm |  | (AL=1.3) |  | 0.3 |  | Internal corrosion of household plumbing systems; erosion of natural deposits; leaching from wood preservatives |  | Copper is an essential nutrient, but some people who drink water containing copper in excess of the action level over a relatively short amount of time may experience gastrointestinal distress. Some people who drink water containing copper in excess of the action level over many years may suffer liver or kidney damage. People with Wilson’s Disease should consult their personal doctor. |
| Cyanide |  | ppb |  | 150 |  | 150 |  | Discharge from steel/metal, plastic and fertilizer factories |  | Some people who drink water containing cyanide in excess of the MCL over many years may experience nerve damage or thyroid problems. |
| Fluoride |  | ppm |  | 2.0 |  | 1 |  | Erosion of natural deposits; water additive which promotes strong teeth; discharge from fertilizer and aluminum factories |  | Some people who drink water containing fluoride in excess of the federal MCL of 4 mg/L over many years may get bone disease, including pain and tenderness of the bones. Children who drink water containing fluoride in excess of the state MCL of 2 mg/L may get mottled teeth. |
| Lead |  | ppb |  | (AL=15) |  | 0.2 |  | Internal corrosion of household water plumbing systems; discharges from industrial manufacturers; erosion of natural deposits |  | Infants and children who drink water containing lead in excess of the action level may experience delays in their physical or mental development. Children may show slight deficits in attention span and learning abilities. Adults who drink this water over many years may develop kidney problems or high blood pressure. |
| Mercury (inorganic) |  | ppb |  | 2 |  | 1.2 |  | Erosion of natural deposits; discharge from refineries and factories; runoff from landfills and cropland |  | Some people who drink water containing mercury in excess of the MCL over many years may experience mental disturbances, or impaired physical coordination, speech and hearing. |
| Nickel |  | ppb |  | 100 |  | 12 |  | Erosion of natural deposits; discharge from metal factories |  | Some people who drink water containing nickel in excess of the MCL over many years may experience liver and heart effects. |
| Nitrate (as nitrate, NO3) |  | ppm |  | 45 |  | 45 |  | Runoff and leaching from fertilizer use; leaching from septic tanks and sewage; erosion of natural deposits |  | Infants below the age of six months who drink water containing nitrate in excess of the MCL may quickly become seriously ill and, if untreated, may die because high nitrate levels can interfere with the capacity of the infant’s blood to carry oxygen. Symptoms include shortness of breath and blueness of the skin. High nitrate levels may also affect the oxygen-carrying ability of the blood of pregnant women. |
| Nitrite (as nitrogen, N) |  | ppm |  | 1 |  | 1 |  | Runoff and leaching from fertilizer use; leaching from septic tanks and sewage; erosion of natural deposits |  | Infants below the age of six months who drink water containing nitrite in excess of the MCL may quickly become seriously ill and, if untreated, may die. Symptoms include shortness of breath and blueness of the skin. |
| Perchlorate |  | ppb |  | 6 |  | 6 |  | Perchlorate is an inorganic chemical used in solid rocket propellant, fireworks, explosives, flares, matches, and a variety of industries. It usually gets into drinking water as a result of environmental contamination from historic aerospace or other industrial operations that used or use, store, or dispose of perchlorate and its salts. |  | Perchlorate has been shown to interfere with uptake of iodide by the thyroid gland, and to thereby reduce the production of thyroid hormones, leading to adverse affects associated with inadequate hormone levels. Thyroid hormones are needed for normal prenatal growth and development of the fetus, as well as for normal growth and development in the infant and child. In adults, thyroid hormones are needed for normal metabolism and mental function. |
| Selenium |  | ppb |  | 50 |  | 30 |  | Discharge from petroleum, glass, and metal refineries; erosion of natural deposits; discharge from mines and chemical manufacturers; runoff from livestock lots (feed additive) |  | Selenium is an essential nutrient. However, some people who drink water containing selenium in excess of the MCL over many years may experience hair or fingernail losses, numbness in fingers or toes, or circulation system problems. |
| Thallium |  | ppb |  | 2 |  | 0.1 |  | Leaching from ore-processing sites; discharge from electronics, glass, and drug factories |  | Some people who drink water containing thallium in excess of the MCL over many years may experience hair loss, changes in their blood, or kidney, intestinal, or liver problems. |
| #### Synthetic Organic Contaminants including Pesticides and Herbicides |  |  |  |  |  |  |  |  |  |  |
| 2,4-D |  | ppb |  | 70 |  | 20 |  | Runoff from herbicide used on row crops, range land, lawns, and aquatic weeds |  | Some people who use water containing the weed killer 2,4-D in excess of the MCL over many years may experience kidney, liver, or adrenal gland problems. |
| 2,4,5-TP (Silvex} |  | ppb |  | 50 |  | 25 |  | Residue of banned herbicide |  | Some people who drink water containing Silvex in excess of the MCL over many years may experience liver problems. |
| Acrylamide |  |  |  | TT |  | (0) |  | Added to water during sewage/wastewater treatment |  | Some people who drink water containing high levels of acrylamide over a long period of time may experience nervous system or blood problems, and may have an increased risk of getting cancer. |
| Alachlor |  | ppb |  | 2 |  | 4 |  | Runoff from herbicide used on row crops |  | Some people who use water containing alachlor in excess of the MCL over many years may experience eye, liver, kidney, or spleen problems, or experience anemia, and may have an increased risk of getting cancer. |
| Atrazine |  | ppb |  | 1 |  | 0.15 |  | Runoff from herbicide used on row crops and along railroad and highway right-of-ways |  | Some people who use water containing atrazine in excess of the MCL over many years may experience cardiovascular system problems or reproductive difficulties. |
| Bentazon |  | ppb |  | 18 |  | 200 |  | Runoff/leaching from herbicide used on beans, peppers, corn, peanuts, rice, and ornamental grasses |  | Some people who drink water containing bentazon in excess of the MCL over many year may experience prostate and gastrointestinal effects. |
| Benzo(a)pyrene (PAH) |  | ppt |  | 200 |  | 7 |  | Leaching from linings of water storage tanks and distribution mains |  | Some people who use water containing benzo(a)pyrene in excess of the MCL over many years may experience reproductive difficulties and may have an increased risk of getting cancer. |
| Carbofuran |  | ppb |  | 18 |  | 1.7 |  | Leaching of soil fumigant used on rice and alfalfa, and grape vineyards |  | Some people who use water containing carbofuran in excess of the MCL over many years may experience problems with their blood, or nervous or reproductive system problems. |
| Chlordane |  | ppt |  | 100 |  | 30 |  | Residue of banned insecticide |  | Some people who use water containing chlordane in excess of the MCL over many years may experience liver or nervous system problems, and may have an increased risk of getting cancer. |
| Dalapon |  | ppb |  | 200 |  | 790 |  | Runoff from herbicide used on rights-of-ways, and crops and landscape maintenance |  | Some people who drink water containing dalapon in excess of the MCL over many years may experience minor kidney changes. |
| Di(2-ethylhexyl) adipate |  | ppb |  | 400 |  | 200 |  | Discharge from chemical factories |  | Some people who drink water containing di(2-ethylhexyl) adipate in excess of the MCL over many years may experience weight loss, liver enlargement, or possible reproductive difficulties. |
| Di(2-ethylhexyl) phthalate |  | ppb |  | 4 |  | 12 |  | Discharge from rubber and chemical factories; inert ingredient in pesticides |  | Some people who use water containing di(2-ethylhexyl) phthalate in excess of the MCL over many years may experience liver problems or reproductive difficulties, and may have an increased risk of getting cancer. |
| Dibromochloropropane (DBCP) |  | ppt |  | 200 |  | 1.7 |  | Banned nematocide that may still be present in soils due to runoff/leaching from former use on soybeans, cotton, vineyards, tomatoes, and tree fruit |  | Some people who use water containing DBCP in excess of the MCL over many years may experience reproductive difficulties and may have an increased risk of getting cancer. |
| Dinoseb |  | ppb |  | 7 |  | 14 |  | Runoff from herbicide used on soybeans, vegetables, and fruits |  | Some people who drink water containing dinoseb in excess of the MCL over many years may experience reproductive difficulties. |
| Dioxin (2,3,7,8-TCDD) |  | ppq (parts per quadrillion) |  | 30 |  | 0.05 |  | Emissions from waste incineration and other combustion; discharge from chemical factories |  | Some people who use water containing dioxin in excess of the MCL over many years may experience reproductive difficulties and may have an increased risk of getting cancer. |
| Diquat |  | ppb |  | 20 |  | 15 |  | Runoff from herbicide use for terrestrial and aquatic weeds |  | Some people who drink water containing diquat in excess of the MCL over many years may get cataracts. |
| Endothall |  | ppb |  | 100 |  | 580 |  | Runoff from herbicide use for terrestrial and aquatic weeds; defoliant |  | Some people who drink water containing endothall in excess of the MCL over many years may experience stomach or intestinal problems. |
| Endrin |  | ppb |  | 2 |  | 1.8 |  | Residue of banned insecticide and rodenticide |  | Some people who drink water containing endrin in excess of the MCL over many years may experience liver problems. |
| Epichlorohydrin |  |  |  | TT |  | (0) |  | Discharge from industrial chemical factories; impurity of some water treatment chemicals |  | Some people who drink water containing high levels of epichlorohydrin over a long period of time may experience stomach problems, and may have an increased risk of getting cancer. |
| Ethylene dibromide (EDB) |  | ppt |  | 50 |  | 10 |  | Discharge from petroleum refineries; underground gas tank leaks; banned nematocide that may still be present in soils due to runoff and leaching from grain and fruit crops |  | Some people who use water containing ethylene dibromide in excess of the MCL over many years may experience liver, stomach, reproductive system, or kidney problems, and may have an increased risk of getting cancer. |
| Glyphosate |  | ppb |  | 700 |  | 900 |  | Runoff from herbicide use |  | Some people who drink water containing glyphosate in excess of the MCL over many years may experience kidney problems or reproductive difficulties. |
| Heptachlor |  | ppt |  | 10 |  | 8 |  | Residue of banned insecticide |  | Some people who use water containing heptachlor in excess of the MCL over many years may experience liver damage and may have an increased risk of getting cancer. |
| Heptachlor epoxide |  | ppt |  | 10 |  | 6 |  | Breakdown of heptachlor |  | Some people who use water containing heptachlor epoxide in excess of the MCL over many years may experience liver damage, and may have an increased risk of getting cancer. |
| Hexachlorobenzene |  | ppb |  | 1 |  | 0.03 |  | Discharge from metal refineries and agricultural chemical factories; byproduct of chlorination reactions in wastewater |  | Some people who drink water containing hexachlorobenzene in excess of the MCL over many years may experience liver or kidney problems, or adverse reproductive effects, and may have an increased risk of getting cancer. |
| Hexachlorocyclopentadiene |  | ppb |  | 50 |  | 50 |  | Discharge from chemical factories |  | Some people who use water containing hexachlorocyclopentadiene in excess of the MCL over many years may experience kidney or stomach problems. |
| Lindane |  | ppt |  | 200 |  | 32 |  | Runoff/leaching from insecticide used on cattle, lumber, gardens |  | Some people who drink water containing lindane in excess of the MCL over many years may experience kidney or liver problems. |
| Methoxychlor |  | ppb |  | 30 |  | 0.09 |  | Runoff/leaching from insecticide used on fruits, vegetables, alfalfa, livestock |  | Some people who drink water containing methoxychlor in excess of the MCL over many years may experience reproductive difficulties. |
| Molinate (Ordram) |  | ppb |  | 20 |  | 1 |  | Runoff/leaching from herbicide used on rice |  | Some people who use water containing molinate in excess of the MCL over many years may experience reproductive effects. |
| Oxamyl (Vydate) |  | ppb |  | 50 |  | 26 |  | Runoff/leaching from insecticide used on field crops, fruits and ornamentals, especially apples, potatoes, and tomatoes |  | Some people who drink water containing oxamyl in excess of the MCL over many years may experience slight nervous system effects. |
| PCBs (Polychlorinated biphenyls) |  | ppt |  | 500 |  | 90 |  | Runoff from landfills; discharge of waste chemicals |  | Some people who drink water containing PCBs in excess of the MCL over many years may experience changes in their skin, thymus gland problems, immune deficiencies, or reproductive or nervous system difficulties, and may have an increased risk of getting cancer. |
| Pentachlorophenol |  | ppb |  | 1 |  | 0.3 |  | Discharge from wood preserving factories, cotton and other insecticidal/herbicidal uses |  | Some people who use water containing pentachlorophenol in excess of the MCL over many years may experience liver or kidney problems, and may have an increased risk of getting cancer. |
| Picloram |  | ppb |  | 500 |  | 500 |  | Herbicide runoff |  | Some people who drink water containing picloram in excess of the MCL over many years may experience liver problems. |
| Simazine |  | ppb |  | 4 |  | 4 |  | Herbicide runoff |  | Some people who use water containing simazine in excess of the MCL over many years may experience blood problems. |
| Thiobencarb |  | ppb |  | 70 |  | 70 |  | Runoff/leaching from herbicide used on rice |  | Some people who use water containing thiobencarb in excess of the MCL over many years may experience body weight and blood effects. |
| Toxaphene |  | ppb |  | 3 |  | 0.03 |  | Runoff/leaching from insecticide used on cotton and cattle |  | Some people who use water containing toxaphene in excess of the MCL over many years may experience kidney, liver, or thyroid problems, and may have an increased risk of getting cancer. |
| ## Volatile Organic Contaminants |  |  |  |  |  |  |  |  |  |  |
| Benzene |  | ppb |  | 1 |  | 0.15 |  | Discharge from plastics, dyes and nylon factories; leaching from gas storage tanks and landfills |  | Some people who use water containing benzene in excess of the MCL over many years may experience anemia or a decrease in blood platelets, and may have an increased risk of getting cancer. |
| Carbon tetrachloride |  | ppt |  | 500 |  | 100 |  | Discharge from chemical plants and other industrial activities |  | Some people who use water containing carbon tetrachloride in excess of the MCL over many years may experience liver problems and may have an increased risk of getting cancer. |
| 1,2-Dichlorobenzene |  | ppb |  | 600 |  | 600 |  | Discharge from industrial chemical factories |  | Some people who drink water containing 1,2-dichlorobenzene in excess of the MCL over many years may experience liver, kidney, or circulatory system problems. |
| 1,4-Dichlorobenzene |  | ppb |  | 5 |  | 6 |  | Discharge from industrial chemical factories |  | Some people who use water containing 1.4-dichlorobenzene in excess of the MCL over many years may experience anemia, liver, kidney, or spleen damage, or changes in their blood. |
| 1,1-Dichloroethane |  | ppb |  | 5 |  | 3 |  | Extraction and degreasing solvent; used in the manufacture of pharmaceuticals, stone, clay, and glass products; fumigant |  | Some people who use water containing 1,1-dichloroethane in excess of the MCL over many years may experience nervous system or respiratory problems. |
| 1,2-Dichloroethane |  | ppt |  | 500 |  | 400 |  | Discharge from industrial chemical factories |  | Some people who use water containing 1,2- dichloroethane in excess of the MCL over many years may have an increased risk of getting cancer. |
| 1,1-Dichloroethylene |  | ppb |  | 6 |  | 10 |  | Discharge from industrial chemical factories |  | Some people who use water containing 1,1-dichloroethylene in excess of the MCL over many years may experience liver problems. |
| cis-1,2-Dichloroethylene |  | ppb |  | 6 |  | 100 |  | Discharge from industrial chemical factories; major biodegradation byproduct of TCE and PCE groundwater contamination |  | Some people who use water containing cis-1,2-dichloroethylene in excess of the MCL over many years may experience liver problems. |
| trans-1,2-Dichloroethylene |  | ppb |  | 10 |  | 60 |  | Discharge from industrial chemical factories; minor biodegradation byproduct of TCE and PCE groundwater contamination |  | Some people who drink water containing trans-1,2-dichloroethylene in excess of the MCL over many years may experience liver problems. |
| Dichloromethane |  | ppb |  | 5 |  | 4 |  | Discharge from pharmaceutical and chemical factories; insecticide |  | Some people who drink water containing dichloromethane in excess of the MCL over many years may experience liver problems and may have an increased risk of getting cancer. |
| 1,2-Dichloropropane |  | ppb |  | 5 |  | 0.5 |  | Discharge from industrial chemical factories; primary component of some fumigants |  | Some people who use water containing 1,2-dichloropropane in excess of the MCL over many years may have an increased risk of getting cancer. |
| 1,3-Dichloropropene |  | ppt |  | 500 |  | 200 |  | Runoff/leaching from nematocide used on croplands |  | Some people who use water containing 1,3-dichloropropene in excess of the MCL over many years may have an increased risk of getting cancer. |
| Ethylbenzene |  | ppb |  | 300 |  | 300 |  | Discharge from petroleum refineries; industrial chemical factories |  | Some people who use water containing ethylbenzene in excess of the MCL over many years may experience liver or kidney problems. |
| Methyl-tert-butyl ether |  | ppb |  | 13 |  | 13 |  | Leaking underground storage tanks; discharges from petroleum and chemical factories |  | Some people who use water containing methyl-tert-butyl ether in excess of the MCL over many years may have an increased risk of getting cancer. |
| Monochlorobenzene |  | ppb |  | 70 |  | 200 |  | Discharge from industrial and agricultural chemical factories and drycleaning facilities |  | Some people who use water containing monochlorobenzene in excess of the MCL over many years may experience liver or kidney problems. |
| Styrene |  | ppb |  | 100 |  | 0.5 |  | Discharge from rubber and plastic factories; leaching from landfills |  | Some people who drink water containing styrene in excess of the MCL over many years may experience liver, kidney, or circulatory system problems. |
| 1,1,2,2-Tetrachloroethane |  | ppb |  | 1 |  | 0.1 |  | Discharge from industrial and agricultural chemical factories; solvent used in production of TCE, pesticides, varnish and lacquers |  | Some people who drink water containing 1,1,2,2-tetrachloroethane in excess of the MCL over many years may experience liver or nervous system problems. |
| Tetrachloroethylene (PCE) |  | ppb |  | 5 |  | 0.06 |  | Discharge from factories, dry cleaners, and auto shops (metal degreaser) |  | Some people who use water containing tetrachloroethylene in excess of the MCL over many years may experience liver problems, and may have an increased risk of getting cancer. |
| 1,2,4-Trichlorobenzene |  | ppb |  | 5 |  | 5 |  | Discharge from textile-finishing factories |  | Some people who use water containing 1,2,4-trichlorobenzene in excess of the MCL over many years may experience adrenal gland changes. |
| 1,1,1-Trichloroethane |  | ppb |  | 200 |  | 1000 |  | Discharge from metal degreasing sites and other factories; manufacture of food wrappings |  | Some people who use water containing 1,1,1-trichloroethane in excess of the MCL over many years may experience liver, nervous system, or circulatory system problems. |
| 1,1,2-Trichloroethane |  | ppb |  | 5 |  | 0.3 |  | Discharge from industrial chemical factories |  | Some people who use water containing 1,1,2- trichloroethane in excess of the MCL over many years may experience liver, kidney, or immune system problems. |
| Trichloroethylene (TCE) |  | ppb |  | 5 |  | 1.7 |  | Discharge from metal degreasing sites and other factories |  | Some people who use water containing trichloroethylene in excess of the MCL over many years may experience liver problems and may have an increased risk of getting cancer. |
| Toluene |  | ppb |  | 150 |  | 150 |  | Discharge from petroleum and chemical factories; underground gas tank leaks |  | Some people who use water containing toluene in excess of the MCL over many years may experience nervous system, kidney, or liver problems. |
| Trichlorofluoromethane |  | ppb |  | 150 |  | 700 |  | Discharge from industrial factories; degreasing solvent; propellant and refrigerant |  | Some people who use water containing trichlorofluoromethane in excess of the MCL over many years may experience liver problems. |
| 1,1,2-Trichloro-1,2,2-trifluoroethane |  | ppm |  | 1.2 |  | 4 |  | Discharge from metal degreasing sites and other factories; drycleaning solvent; refrigerant |  | Some people who use water containing 1,1,2-trichloro-1,2,2-trifloroethane in excess of the MCL over many years may experience liver problems. |
| Vinyl chloride |  | ppt |  | 500 |  | 50 |  | Leaching from PVC piping; discharge from plastics factories; biodegradation byproduct of TCE and PCE groundwater contamination |  | Some people who use water containing vinyl chloride in excess of the MCL over many years may have an increased risk of getting cancer. |
| Xylenes |  | ppm |  | 1.750 |  | 1.8 |  | Discharge from petroleum and chemical factories; fuel solvent |  | Some people who use water containing xylenes in excess of the MCL over many years may experience nervous system damage. |
| ### Disinfection Byproducts, Disinfectant Residuals, and Disinfection Byproduct Precursors |  |  |  |  |  |  |  |  |  |  |
| TTHMs (Total Trihalomethanes) |  | ppb |  | 80 |  | N/A |  | By-product of drinking water disinfection |  | Some people who drink water containing trihalomethanes in excess of the MCL over many years may experience liver, kidney, or central nervous system problems, and may have an increased risk of getting cancer. |
| Haloacetic Acids |  | ppb |  | 60 |  | N/A |  | Byproduct of drinking water disinfection |  | Some people who drink water containing haloacetic acids in excess of the MCL over many years may have an increased risk of getting cancer. |
| Bromate |  | ppb |  | 10 |  | 0.1 |  | Byproduct of drinking water disinfection |  | Some people who drink water containing bromate in excess of the MCL over many years may have an increased risk of getting cancer. |
| Chloramines |  | ppm |  | [MRDL = 4.0 (as Cl2)] |  | [MRDLG = 4 (as Cl2)] |  | Drinking water disinfectant added for treatment |  | Some people who use water containing chloramines well in excess of the MRDL could experience irritating effects to their eyes and nose. Some people who drink water containing chloramines well in excess of the MRDL could experience stomach discomfort or anemia. |
| Chlorine |  | ppm |  | [MRDL = 4.0 (as Cl2)] |  | [MRDLG = 4 (as Cl2) |  | Drinking water disinfectant added for treatment |  | Some people who use water containing chlorine well in excess of the MRDL could experience irritating effects to their eyes and nose. Some people who drink water containing chlorine well in excess of the MRDL could experience stomach discomfort. |
| Chlorite |  | ppm |  | 1.0 |  | 0.05 |  | Byproduct of drinking water disinfection |  | Some infants and young children who drink water containing chlorite in excess of the MCL could experience nervous system effects. Similar effects may occur in fetuses of pregnant women who drink water containing chlorite in excess of the MCL. Some people may experience anemia. |
| Chlorine Dioxide |  | ppb |  | [MRDL = 800 (as ClO2)] |  | [MRDLG = 800 (as ClO2)] |  | Drinking water disinfectant added for treatment |  | Some infants and young children who drink water containing chlorine dioxide in excess of the MRDL could experience nervous system effects. Similar effects may occur in fetuses of pregnant women who drink water containing chlorine dioxide in excess of the MRDL. Some people may experience anemia. |
| Control of DBP precursors (TOC) |  |  |  | TT |  | N/A |  | Various natural and man-made sources |  | Total organic carbon (TOC) has no health effects. However, total organic carbon provides a medium for the formation of disinfection byproducts. These byproducts include trihalomethanes (THMs) and haloacetic acids (HAAs). Drinking water containing these byproducts in excess of the MCL may lead to adverse health effects, liver or kidney problems, or nervous system effects, and may lead to an increased risk of cancer. |

ATTACHMENT 2

Regulated Contaminants with SECONDARY DRINKING WATER STANDARDS (a)

Monitoring Required by Section 64449, Chapter 15, Title 22, California Code of Regulations

| Contaminant | Unit Measurement | MCL | Typical Source of Contaminant | Notification Level | Notification Level Health Effects Language (Optional) |
| --- | --- | --- | --- | --- | --- |
| Aluminum | ppb | 200 | Erosion of natural deposits; residual from some surface water treatment processes |  |  |
| Color | Units | 15 | Naturally-occurring organic materials |  |  |
| Copper | ppm | 1.0 | Internal corrosion of household plumbing systems; erosion of natural deposits; leaching from wood preservatives |  |  |
| Foaming Agents (MBAS) | ppb | 500 | Municipal and industrial waste discharges |  |  |
| Iron | ppb | 300 | Leaching from natural deposits; industrial wastes |  |  |
| Manganese | ppb | 50 | Leaching from natural deposits | 500 | The notification level for manganese is used to protect consumers from neurological effects. High levels of manganese in people have been shown to result in effects of the nervous system. |
| Methyl-tert-butyl ether (MTBE) | ppb | 5 | Leaking underground storage tanks; discharge from petroleum and chemical factories |  |  |
| Odor--Threshold | Units | 3 | Naturally-occurring organic materials |  |  |
| Silver | ppb | 100 | Industrial discharges |  |  |
| Thiobencarb | ppb | 1 | Runoff/leaching from rice herbicide |  |  |
| Turbidity | Units | 5 | Soil runoff |  |  |
| Zinc | ppm | 5.0 | Runoff/leaching from natural deposits; industrial wastes |  |  |
| Total Dissolved Solids (TDS) | ppm | 1000 | Runoff/leaching from natural deposits |  |  |
| Specific Conductance | µS/cm | 1600 | Substances that form ions when in water; seawater influence |  |  |
| Chloride | ppm | 500 | Runoff/leaching from natural deposits; seawater influence |  |  |
| Sulfate | ppm | 500 | Runoff/leaching from natural deposits; industrial wastes |  |  |

(a)	There are no PHGs, MCLGs, or mandatory standard health effects language for these constituents because secondary MCLs are set on the basis of aesthetics.

ATTACHMENT 3

State Regulated Contaminants with No Maximum Contaminant Levels
 (i.e., Unregulated Chemicals)

Monitoring Formerly Required by Repealed Section 64450, Chapter 15, Title 22, California Code of Regulations

Results of monitoring under former section 64450 (UCMR) need only be included for 5 years from the date of the last sampling or until any of the detected contaminants becomes regulated and subject to routine monitoring requirement, whichever comes first.  Section 64450 was repealed effective October 18, 2007.

Inclusion of the notification level and health effects language for levels above the notification level is only recommended, not required.

| Chemicals | Notification Level | Health Effects Language (Optional) |
| --- | --- | --- |
| Boron | 1 ppm | The babies of some pregnant women who drink water containing boron in excess of the notification level may have an increased risk of developmental effects, based on studies in laboratory animals. |
| Chromium VI (Hexavalent chromium) | n/a | n/a |
| Dichlorodifluoromethane (Freon 12) | 1 ppm | Some people who drink water containing dichlorodifluoromethane far in excess of the notification level may experience neurological and cardiac effects. Long- term exposures to dichlorodifluoromethane resulted in smaller body weight in laboratory animals. |
| Ethyl-tert-butyl ether (ETBE) | n/a | n/a |
| tert-Amyl-methyl ether (TAME) | n/a | n/a |
| tert-Butyl alcohol (TBA) | 12 ppb | Some people who use water containing tert-butyl alcohol in excess of the notification level over many years may have an increased risk of getting cancer, based on studies in laboratory animals. |
| Trichloropropane (1,2,3-TCP). | 5 ppt | Some people who use water containing 1,2,3-trichloropropane in excess of the notification level over many years may have an increased risk of getting cancer, based on studies in laboratory animals. |
| Vanadium | 50 ppb | The babies of some pregnant women who drink water containing vanadium in excess of the notification level may have an increased risk of developmental effects, based on studies in laboratory animals. |

ATTACHMENT 4

Federal Regulated Contaminants with No Maximum Contaminants Levels
(i.e., Federal UCMR 1 and UCMR 2)

Background

The 1996 Amendments to the Safe Drinking Water Act required the EPA to establish criteria for a monitoring program for unregulated contaminants and to publish a list of contaminants to be monitored.

| UCMR 1 (2001 – 2003 Monitoring) |  | List 1 Assessment Monitoring | List 2 Screening Survey |
| --- | --- | --- | --- |
| In 1999, EPA revised the Unregulated Contaminant Monitoring Rule to incorporate a tiered monitoring approach. UCMR 1 had assessment monitoring (List 1) and screening survey (List 2) components. A randomly selected sample of 800 small water systems serving 10,000 or fewer people conducted List 1 - Assessment Monitoring over a 12-month period between 2001 and 2003. A subset of the 800 small water systems were also required to participate in a List 2 - Screening Survey in 2001 (chemical contaminants) and 2003 (Aeromonas). Small system monitoring was paid for by the EPA, including provisions for sampling equipment and sample shipping, testing, and analysis. The State Drinking Water Agency or the EPA informed you if your system was selected. |  | 2,4-dinitrotoluene 2,6-dinitrotoluene Acetochlor DCPA mono-acid degradate DCPA di-acid degradate 4,4’ – DDE EPTC Molinate MTBE Nitrobenzene Perchlorate Terbacil | 1,2-diphenylhydrazine 2-methyl-phenol 2,4-dichlorophenol 2,4-dinitrophenol 2,4,6-trichlorophenol Aeromonas Alachlor ESA Diazinon Disulfoton Diuron Fonofos Linuron Nitrobenzene Prometon RDX Terbufos |
| UCMR 2 (2008 – 2010 Monitoring) |  | List 1 Assessment Monitoring | List 2 Screening Survey |
| In 2007, EPA revised the Unregulated Contaminant Monitoring Rule to establish a new set of unregulated contaminants. A randomly selected sample of 800 small water systems serving 10,000 or fewer people is required to conduct List 1 - Assessment Monitoring over a 12-month period from January 2008 – December 2010. A subset of the 800 small water systems are also required to participate in a List 2 - Screening Survey over a 12-month period from January 2008 – December 2010. Small system monitoring is paid for by the EPA, including provisions for sampling equipment and sample shipping, testing, and analysis. The State Drinking Water Agency or the EPA informed you if your system was selected. |  | Dimethoate Terbufos sulfone 2,2',4,4'-tetrabromodiphenyl ether 2,2',4,4',5-pentabromodiphenyl ether 2,2',4,4',5,5'-hexabromobiphenyl 2,2',4,4',5,5'-hexabromodiphenyl ether 2,2',4,4',6-pentabromodiphenyl ether 1,3-dinitrobenzene 2,4,6-trinitrotoluene (TNT) Hexahydro-1,3,5-trinitro-1,3,5-trazine (RDX) | Acetochlor ethane sulfonic acid Acetochlor oxanilic acid Alachlor ethane sulfonic acid Alachlor oxanilic acid Metolachlor ethane sulfonic acid Metolachlor oxanilic acid Acetochlor Alachlor Metolachlor N-nitrosodiethylamine (NDEA) N-nitrosodimethylamine (NDMA) N-nitroso-di-n-butylamine (NDBA) N-nitroso-di-n-propylamine (NDPA) N-nitrosomethylethylamine (NMEA) N-nitrosopyrrolidine (NPYR) |

ATTACHMENT 5

State Contaminants with Notification Levels

Inclusion of the notification level and health effects language for levels above the notification level is only recommended, not required.

| Chemical | Notification Level | Health Effects Language (Optional) |
| --- | --- | --- |
| Boron | 1 ppm | See Attachment 3 |
| n-Butylbenzene | 260 ppb | n/a |
| sec-Butylbenzene | 260 ppb | n/a |
| tert-Butylbenzene | 260 ppb | n/a |
| Carbon disulfide | 160 ppb | n/a |
| Chlorate | 800 ppb | n/a |
| 2-Chlorotoluene | 140 ppb | n/a |
| 4-Chlorotoluene | 140 ppb | n/a |
| Diazinon | 1.2 ppb | n/a |
| Dichlodifluoromethane (Freon 12) | 1 ppm | See Attachment 3 |
| 1,4-Dioxane | 1 ppb | Some people who use water containing 1,4-dioxane in excess of the Notification Level over many years may experience liver or kidney problems and may have an increased risk of getting cancer, based on studies in laboratory animals. |
| Ethylene glycol | 14 ppm | n/a |
| Formaldehyde | 100 ppb | n/a |
| HMX | 350 ppb | n/a |
| Isopropylbenzene | 770 ppb | n/a |
| Manganese | 500 ppb | The notification level for manganese is used to protect consumers from neurological effects. High levels of manganese in people have been shown to result in effects of the nervous system. |
| Methyl isobutyl ketone (MIBK) | 120 ppb | n/a |
| Naphthalene | 17 ppb | n/a |
| N-Nitrosodiethylamine (NDEA) | 10 ppt | n/a |
| N-Nitrosodimethylamine (NDMA) | 10 ppt | n/a |
| N-Nitrosodi-n-propylamine (NDPA) | 10 ppt | n/a |
| Propachlor | 90 ppb | n/a |
| n-Propylbenzene | 260 ppb | n/a |
| RDX | 300 ppt | n/a |
| Tertiary butyl alcohol (TBA) | 12 ppb | See Attachment 3 |
| 1,2,3-Trichloropropane (1,2,3-TCP) | 5 ppt | See Attachment 3 |
| 1,2,4-Trimethylbenzene | 330 ppb | n/a |
| 1,3,5-Trimethylbenzene | 330 ppb | n/a |
| 2,4,6-Trinitrotoluene (TNT) | 1 ppb | n/a |
| Vanadium | 50 ppb | See Attachment 3 |

ATTACHMENT 6

Special Language for Nitrate, Arsenic, Lead, Radon, Cryptosporidium, Ground Water Systems, and Surface Water Systems

(A)	Nitrate:  For systems that detect nitrate (as NO3) above 23 mg/L, but below 45 mg/L, the following language is REQUIRED:

Nitrate in drinking water at levels above 45 mg/L is a health risk for infants of less than six months of age. Such nitrate levels in drinking water can interfere with the capacity of the infant’s blood to carry oxygen, resulting in serious illness; symptoms include shortness of breath and blueness of the skin. Nitrate levels above 45 mg/L may also affect the ability of the blood to carry oxygen in other individuals, such as pregnant women and those with specific enzyme deficiencies. If you are caring for an infant, or you are pregnant, you should ask advice from your health care provider.

If a utility cannot demonstrate to the Department with at least five years of the most current monitoring data that its nitrate levels are stable, it must also add the following language to the preceding statement on nitrate:

Nitrate levels may rise quickly for short periods of time because of rainfall or agricultural activity.

(B)	Arsenic:  For systems that detect arsenic above 5 ppb, but below or equal to 10 ppb, the following language is REQUIRED:

While your drinking water meets the federal and state standard for arsenic, it does contain low levels of arsenic. The arsenic standard balances the current understanding of arsenic’s possible health effects against the cost of removing arsenic from drinking water. The U.S. Environmental Protection Agency continues to research the health effects of low levels of arsenic, which is a mineral known to cause cancer in humans at high concentrations and is linked to other health effects such as skin damage and circulatory problems.

(C)	Lead:  For systems that detect lead above 15 ppb in more than 5%, and up to and including 10%, of sites sampled (or if your system samples fewer than 20 sites and has even one sample above the AL), the following language is REQUIRED:

Infants and young children are typically more vulnerable to lead in drinking water than the general population. It is possible that lead levels at your home may be higher than at other homes in the community as a result of materials used in your home’s plumbing. If you are concerned about elevated lead levels in your home’s water, you may wish to have your water tested and/or flush your tap for 30 seconds to 2 minutes before using tap water. Additional information is available from the USEPA Safe Drinking Water Hotline (1-800-426-4791).

(D)	Radon:  Systems that performed monitoring that indicates the presence of radon in the finished water MUST include the results of the monitoring and an explanation of the significance of the results.  The following language MAY be used:

We constantly monitor the water supply for various contaminants. We have detected radon in the finished water supply in _____ out of _____ samples tested. There is no federal regulation for radon levels in drinking water. Exposure over a long period of time to air transmitting radon may cause adverse health effects.

The language below MAY be included if the level of information is helpful.

Radon is a radioactive gas that you cannot see, taste, or smell. It is found throughout the U.S. Radon can move up through the ground and into a home through cracks and holes in the foundation. Radon can build up to high levels in all types of homes. Radon can also get into indoor air when released from tap water from showering, washing dishes, and other household activities. Compared to radon entering the home through soil, radon entering the home through tap water will in most cases be a small source of radon in indoor air. Radon is a known human carcinogen. Breathing air containing radon can lead to lung cancer. Drinking water containing radon may also cause increased risk of stomach cancer. If you are concerned about radon in your home, test the air in your home. Testing is inexpensive and easy. You should pursue radon removal for your home if the level of radon in your air is 4 picocuries per liter of air (pCi/L) or higher. There are simple ways to fix a radon problem that are not too costly. For additional information, call your State radon program (1-800-745-7236, the EPA Safe Drinking Water Act Hotline (1-800-426-4791), or the National Safe Council Radon Hotline (1-800-SOS-RADON).

(E)	Cryptosporidium:  Systems that have performed any monitoring for Cryptosposidium that indicates that Cryptosporidium may be present in the source water or finished water MUST include the results of the monitoring and an explanation of the significance of the results.  The following language MAY be used:

Cryptosporidium is a microbial pathogen found in surface water throughout the U.S. Although filtration removes Cryptosporidium, the most commonly-used filtration methods cannot guarantee 100 percent removal. Our monitoring indicates the presence of these organisms in our source water and/or finished water. Current test methods do not allow us to determine if the organisms are dead or if they are capable of causing disease. Ingestion of Cryptosporidium may cause cryptosporidiosis, an abdominal infection. Symptoms of infection include nausea, diarrhea, and abdominal cramps. Most healthy individuals can overcome the disease within a few weeks. However, immuno-compromised people, infants, small children, and the elderly are at greater risk of developing life-threatening illness. We encourage immuno-compromised individuals to consult their doctor regarding appropriate precautions to take to avoid infection. Cryptosporidium must be ingested to cause disease, and it may be spread through means other than drinking water.

(F)	Additional Special Language for Lead:  For community water systems, the following language is REQUIRED:

If present, elevated levels of lead can cause serious health problems, especially for pregnant women and young children. Lead in drinking water is primarily from materials and components associated with service lines and home plumbing. [INSERT NAME OF UTILITY] is responsible for providing high quality drinking water, but cannot control the variety of materials used in plumbing components. When your water has been sitting for several hours, you can minimize the potential for lead exposure by flushing your tap for 30 seconds to 2 minutes before using water for drinking or cooking. If you are concerned about lead in your water, you may wish to have your water tested. Information on lead in drinking water, testing methods, and steps you can take to minimize exposure is available from the Safe Drinking Water Hotline or at http://www.epa.gov/safewater/lead.

(G)	Ground Water Systems:  For ground water systems that had a TT violation described in Item S of the SWS CCR Form Instructions, the following language MAY be used to describe the potential health effects.  USEPA did not provide standard health effect language for these TT violations in the Federal Ground Water Rule; USEPA provided the language in their guidance to water systems.

Inadequately protected or treated water may contain disease-causing organisms. These organisms can cause symptoms such as diarrhea, nausea, cramps, and associated headaches.

(H)	Surface Water Systems: For surface water systems that had a TT violation under the SWTR, IESWTR, FBRR, or Federal LT1ESWTR , the following language is REQUIRED to describe the potential health effects:

Inadequately treated water may contain disease-causing organisms. These organisms include bacteria, viruses, and parasites that can cause symptoms such as nausea, cramps, diarrhea, and associated headaches.

For surface water systems that had a TT violation under the Federal LT2ESWTR, the following language MAY be used to describe the potential health effects.  USEPA did not provide standard health effect language for these TT violations in the Federal LT2ESWTR; USEPA provided the language in their guidance to water systems.

| Federal LT2ESWTR TT Violation | Health Effect Language |
| --- | --- |
| Uncovered and Untreated Finished Water Reservoir | Inadequately protected water may contain disease-causing organisms. These organisms can cause symptoms such as diarrhea, nausea, cramps, and associated headaches. |
| Determine and Report Bin Classification | Inadequately treated water may contain disease-causing organisms. These organisms can cause symptoms such as diarrhea, nausea, cramps, and associated headaches. |
| Provide or Install an Additional Level of Treatment | Inadequately treated water may contain disease-causing organisms. These organisms can cause symptoms such as diarrhea, nausea, cramps, and associated headaches. |